SCHOOL OF MATERIALS SCIENCE & ENGINEERING

CONTINUOUS ASSESSMENT 1 SEMESTER 1 2024-2025

MS1016 – THERMODYNAMICS OF MATERIALS

(Answer Sheet)

25 September 2024	Time Allowed: 1 hour

INSTRUCTIONS

1.	This paper contains THREE (3) short questions and TEN (10) MCQ questions.

2.	Answer ALL questions.

3. 	Write all answers on the answer sheet provided and clearly write your name and matriculation number on it.

4.	This is an open-notes test.

5.	All questions have ONLY ONE correct answer for the MCQ questions.

6.	You may retain this question sheet. Only answers on the submitted answer sheet will be considered.

PART I – SHORT QUESTIONS

- The linear coefficient of thermal expansion, αl, is one-third of the volumetric coefficient of thermal expansion, αv. The volumetric coefficient, αv, is defined by:

αv=1V∂V∂TP	and 	-∂S∂PT=∂V∂TP

Given that the linear thermal expansion of gold is 14.2×10-6K-1, determine the change in specific entropy of gold when the pressure increases from 1 atm to 10 atm. Consider 1atm=1.013×105N/m2. The atomic weight of gold is 196.97 g/mol and its density is 19.28 g/cm³

(20marks)

1. Determine the Volumetric Coefficient of Thermal Expansion:

αv=3αl=3×14.2×10-6 K-1=42.6×10-6 K-1		(3 marks)

2. Calculate the Specific Volume:

V=Atomic weightDensity

V=196.97 g/mol19.28 g/cm3=10.2 cm3/mol		(4 marks)

3. Using the relation between entropy, volume, and temperature coefficients:

-∂S∂PT=∂V∂TP=αvV			(1 mark)

αvV=-42.6×10-6 K-1×10.2 cm3/mol		(1 mark)

αvV=-434.52×10-6 cm3K-1mol-1		(1 mark)

4. Finally, determine the entropy change for the pressure range:

S2-S1=12dS=-P1P2∂V∂TP=P1P2αvVdP		(5 marks)

For a condensed solid phase, V is constant. Therefore, the specific entropy change:

S2-S1=-αvVΔP=-434.52×10-6 cm3K-1mol-1×9 ×1.013×105N/m2

(4 marks)

ΔS=-3.962×10-4 JK-1mol-1			(1 mark)

- a. Give a brief description of the terms, ‘reversible’ and ‘irreversible’ work in Thermodynamics in terms of entropy. Explain the effect of irreversible work on the efficiency of real-world systems and its relation to lost work. (8 marks)

Reversible work is defined as work performed under equilibrium conditions, implying a quasi-static process where the system can be returned to its initial state without leaving any net change in the system or its surroundings. In a reversible process, there is no change in entropy				 (2 marks)

For reversible work, dS = 0

Irreversible work, on the other hand, occurs when the system cannot be returned to its initial state without external intervention. Irreversible processes always lead to an increase in entropy, as stated by the Second Law of Thermodynamics. These processes are spontaneous because they result in an increase in the system's entropy. 								(2 marks)

For irreversible work, dS > 0

The relationship relating reversible work and irreversible work can be modelled as:

dSIRREVERSIBLE = dSREVERSIBLE +Lost Work

This equation indicates that irreversible work leads to lost work, which reduces the efficiency of real-world systems. The increase in entropy associated with irreversible work represents energy that cannot be converted into useful work, thereby lowering the overall efficiency of the system.		(4 marks)

b. Considering a Carnot engine having a thermal efficiency of 0.6 and its hot reservoir is at 600K.

  - What is the temperature of the cold reservoir? Also, what are some assumptions that must be made about the Carnot engine? (6 marks)

Assume reversible heat flow and work, steady state, and a closed system engine. 							(3 marks)

The efficiency of a Carnot engine is given as:

-δWδQH =[TH- TL]TH= ηHE

Substituting thermal efficiency of 0.6 and its hot reservoir is at 600K, we can calculate TL:

600- TL600= 0.6

TL= 240K				(3 marks)

  - What is the work output of the engine per cycle if 3000J of heat is rejected to the cold reservoir? (6 marks)

δQHTH+δQLTL= 0

δQHTH= -δQLTL

Using QL = -3000 J (negative because this is the heat given by the system to the cold reservoir), we get:

δQH600= --3000240

QH = 7500J				(3 marks)

By the First Law, δQ+δW=0

Thus, δQH+ δQL +δW=0

δW= -δQH- δQL

Therefore, the work done on the engine is:

δW= --3000- 7500= -4500J

The net work done on the system is negative, which means that the engine performs 4500J work per cycle.		(3 marks)

- Steam enters a converging-diverging nozzle as shown in the Fig. below, operating at steady state with P1, = 40 bar, T1= 400°C and a velocity of 10 m/s. The steam flows through the nozzle with negligible heat transfer and no significant change in potential energy. At the outlet, P2= 15 bar and the velocity is 665 m/s. The mass flow rate is 2 kg/s.

Determine the specific enthalpy of the steam at outlet.

Determine the outlet area of the nozzle, in m2.

(Note: Take specific enthalpy of superheated steam at 40 bar, 400°C as 3213.6 kJ/kg and the specific volume v, at outlet is 0.1627 m3 /kg)

Hint:      Apply the steady state flow equation between 1 and 2:

mi(Hi + KEi + PEi) δmi - mo(Ho + KEo + PEo) δmo + 𝛿Q + 𝛿W = 0

(20 marks)

Solution:

Given Inlet conditions

P1= 40 bar, T1= 4000C

C1= 10 m/s, h1= 3213.6 kJ/kg

Given Outlet conditions

P2=15 bar,

C2=665 m/s

Applying steady state flow equation between 1 and 2 we get:

mi(Hi + KEi + PEi) δmi - mo(Ho + KEo + PEo) δmo + 𝛿Q + 𝛿W = 0    Eq. (1)

Heat Transfer is negligible, 𝛿Q=0

There is no work done by the system, 𝛿W=0

There is one inlet and one outlet and a single steam, m1=m2

There is no change in potential energy also, ΔPE=0

Simplifying the Eq. 1, we get:

h1+C122=h2+C222           Eq. 2 						(10 marks)

Putting the values of C1= 10 m/s, h1= 3213.6 KJ/Kg, C2=665 m/s in Eq. 2 we get

h2= 2992.5 kJ/kg

The specific enthalpy of the steam at outlet is 2992.5 kJ/kg. [Ans]		(5 marks)

From conservation of mass

ṁ=ρ1A1C1=ρ2A2C2

ṁ =A1C1v1=A2C2v2 …Eq. 3

The specific volume v, at outlet is 0.1627 m3 /kg, v2=0.1627 m3/kg

We know ṁ= 2 Kg/s, C2= 665 m/s

Putting all the values in Eq. 3, we get

A2= 4.89 X 10-4 m2

The exit area of the nozzle is 4.89 X 10-4 m2 [Ans.] 				(5 marks)

PART II Multiple Choice Questions

Answer all the TEN MCQ questions below. There is only ONE answer for each question. (4 marks each)

- Which of the following statements best describes the Helmholtz free energy (F)?

- It defines the free energy of a system at constant pressure and temperature.

- it is used to determine spontaneity under conditions of constant temperature and volume.

- it remains constant during phase transitions at constant pressure.

- it increases as the entropy of a system increases.

ANS: B

the rate of it with temperature at constant volume equals to the enthalpy of the system.

Explanation:

- This statement is incorrect. Helmholtz free energy (F) is defined as F=U-TS. It is used to describe the free energy of a system at constant temperature and volume.

- This statement is correct. Helmholtz free energy is particularly useful for determining the spontaneity of processes occurring at constant temperature and volume. A negative change in Helmholtz free energy (∆F) indicates that the process is spontaneous under these conditions.

- This statement is incorrect. Helmholtz free energy (F) does not necessarily remain constant during phase transitions, even at constant pressure, because it is defined for constant temperature and volume conditions. Moreover, phase transitions typically involve changes in both entropy and volume, thus affecting F.

- This statement is incorrect. Helmholtz free energy (F) is given by F=U-TS. If entropy (S) increases while temperature (T) and internal energy (U) are constant, F will decrease.

- This statement is incorrect. According to the definition of Helmholtz free energy F=U-TS, thus ∂F∂TV=-S. The rate of change in F with respect to T at constant volume is actually related to entropy (S), not enthalpy (H).

- For an ideal gas, what is entropy change when its pressure changes from 2 atm to 20 atm? Given that the ideal gas constant is 8.314 J/(mol∙K).

- 5.76 J/(mol∙K)

- 19.14 J/(mol∙K)

ANS: B

24.03 J/(mol∙K)

- 24.91 J/(mol∙K)

- 25.70 J/(mol∙K)

Explanation:

For an idea gas, PV=RT, hence ∂V∂TP= RP

According to the Maxwell Relations of G, ∂S∂PT=-∂V∂TP=-RP,

Or dS=-RPdP

For a change in pressure from 2 atm to 20 atm,

S2-S1=-P1P2-RPdP=Rln20-Rln2=Rln10=-19.14 J/(mol∙K)

- Which of the following statements is not correct?

ANS: D

For the same mass of water and ice under identical conditions, the entropy of water is larger compared to ice.

- A spontaneous chemical reaction is not always rapid.

- For reaction CaCO3 (solid) → CaO (solid) + CO2 (gas), whether it can proceed spontaneously depends on the temperature.

- At room temperature, the reaction C (solid) + CO2 (gas) → 2CO (gas) cannot proceed spontaneously, which indicates that the enthalpy change of this reaction is negative.

- Considering an isentropic process, the entropy remains constant, and no heat is transferred across the system.

Explanation:

- This statement is correct. Entropy is a measure of disorder or randomness in a system. Liquid water is more disordered compared to solid ice, which has a structured and crystalline arrangement. Thus, the entropy of water is higher compared to ice at the same pressure and temperature.

- This statement is correct. Spontaneity refers to the thermodynamic favorability of a reaction, indicated by a negative Gibbs free energy change. It does not provide information about the reaction rate. Some spontaneous reactions can be extremely low due to high activation energy barrier, such as the conversion of diamond to graphite.

- This statement is correct. The spontaneity of this decomposition reaction depends on the temperature because it involves a significant change in entropy due to the production of a gas (CO2). At higher temperatures, the entropy contribution (-T∆S) becomes significant, potentially making the reaction spontaneous even if enthalpy change (∆H) is positive.

- This statement is not correct. The non-spontaneity of this reaction at room temperature indicates that the Gibbs free energy (∆G=∆H-T∆S) is positive. Meanwhile, since there is more gas produced in the reaction, the entropy contribution ∆S is positive. To ensure ∆G is positive, the enthalpy contribution ∆H should be positive.

- This statement is correct. An isentropic process is characterized by constant entropy, which implies that it is both adiabatic (no heat transfer) and reversible. Thus, in an isentropic process, entropy does not change, and no heat is exchanged with the surroundings.

- For an irreversible process, comparing the actual work done on a gas with the reversible work done, which relation below is most likely to be correct?

ANS: A

W>Wrev

  - W<Wrev

  - |W|>|Wrev|

  - |W|<|Wrev|

  - None of the above

Explanation:

For a compression process, the work required in a reversible process (Wrev) is always less than the work required in an irreversible process (W). This is because the irreversible process involves additional losses due to factors like friction or turbulence, which are not present in a reversible process.

- Which of the following is true?

  - It is impossible to transfer heat from a cooler to a hotter body under ANY conditions

  - It is impossible to perform a process whose sole purpose is the conversion of heat into an equivalent amount of work

  - In an irreversible compression process of an ideal gas, the lost work (W-Wrev) is always negative

  - The 2nd law of thermodynamics is a consequence of the 1st law of thermodynamics

ANS: B

All of the above statements are false

Explanation:

A) It is impossible to transfer heat from a cooler to a hotter body under ANY conditions.

- False. It is indeed possible to transfer heat from a cooler body to a hotter one, but not spontaneously. According to the second law of thermodynamics, this process requires external work.

B) It is impossible to perform a process whose sole purpose is the conversion of heat into an equivalent amount of work.

- True. This statement reflects the second law of thermodynamics, which states that no heat engine operating in a cycle can convert all the heat absorbed into work. There will always be some heat rejected to a cooler reservoir.

C) In an irreversible expansion process of an ideal gas, the lost work (W - Wrev) is always negative.

- False. For a compression process, the work required in a reversible process (Wrev) is always less than the work required in an irreversible process (W). This is because the irreversible process involves additional losses due to factors like friction or turbulence, which are not present in a reversible process.

D) The 2nd law of thermodynamics is a consequence of the 1st law of thermodynamics.

- False. The second law of thermodynamics introduces the concept of entropy and dictates the direction of thermodynamic processes, while the first law is about energy conservation. The second law is not a consequence of the first; rather, it adds supplements our understanding of thermodynamic processes.

- During lecture, we learnt that the reaction Ag + ½ Cl2 = AgCl yields an entropy change of -57.7 J(mol.K)-1. What does this tell us?

ANS: B

The reaction is non-spontaneous at all temperatures.

  - The disorder of the system decreases during the reaction.

  - The reaction absorbs heat from the surroundings.

  - The reaction must be accompanied by a positive enthalpy change.

  - All of the above statements are true.

Explanation:

As per the second law of thermodynamics, A negative entropy change means that the disorder or randomness of the system decreases. In this reaction, as a solid (Ag) and a gas (Cl₂) form a solid salt (AgCl), the system becomes more ordered.

- A small container has gas at high pressure. It is placed in an evacuated space. If the container is punctured, work done by the gas is

- Positive

ANS: D

Cannot be determined

- Infinity

- Zero

- None of the above

Explanation:

When the container is punctured, the gas expands into the evacuated space. Since the external pressure in the evacuated space is effectively zero, the work done by the gas during this expansion can be calculated using the formula:

W=-∫Pext dV

where Pext​ is the external pressure. In this case, since Pext is zero, the work done by the gas will also be zero. It is an example of Free expansion.

Therefore, the correct answer is D) Zero

- A cylinder contains 5 m3 of an ideal gas at a pressure of 1 bar. This gas is compressed in a reversible isothermal process till its pressure increases to 5 bar. The work in kJ required for this process

- 804.7

ANS: A

953.2

- 981.7

- 1012.2

- 563.1

Explanation:

P1=1 bar, P2=5 bar, V1= 5 m3

W=-PⅆV

Isothermal Process

PV=C (Where C is a constant) ; P1V1=P2V2=C= 105 X 5 Pa.m3

V2=1 m3

P=C∕V

W=-∫CVⅆV

W=-ClnV2V1

W= -105 X 5 X ln15

W=-105 X 5 X (-1.6094) J

W=-102 X 5 X (-1.6094) KJ

W= 804.718 KJ [Option A]

- A system is partitioned into two equal parts. An intensive property of each part will

- Become half

ANS: B

Remains unchanged

- double

- Depend on the nature of the intensive property

- None of the above

Explanation:

An intensive property is one that does not depend on the amount of matter or the size of the system. Examples include temperature, pressure, and density. Since these properties are independent of the size or volume of the system, partitioning the system into two equal parts does not change the value of these properties for each part.

Therefore, the correct answer is: B) Remains unchanged

ANS: C

A perfect gas, undergoes isentropic process then which of the following is correct?

- PVγ-1=Constant

- TP(γ-1)/γ=Constant

- TVγ-1=Constant

- All of the options in A), B), C)

- None of the above

Explanation:

PV=mRTP=mRTV

mRTVVγ=constant

TVVγ=constant

TVγ-1=constant [Option C]

For an ideal gas undergoing an adiabatic process

PVγ = constant

PV=mRTV=mRTP

PRTPγ=constant

PPγTγ=constant

P1-γTγ=constant

TP1-γγ=constant

PVγ = constant