SCHOOL OF MATERIALS SCIENCE & ENGINEERING

CONTINUOUS ASSESSMENT 1 SEMESTER 1 2025-2026

MS1016 – THERMODYNAMICS OF MATERIALS

(Answer Sheet)

17 September 2025	Time Allowed: 1 hour

INSTRUCTIONS

1.	This paper contains FOUR (4) short answer questions

2.	Answer ALL questions.

3. 	Write all answers on the answer sheet provided and clearly write your name and matriculation number on it.

4.	This is an open-notes test.

5.	You may retain this question sheet. Only answers on the submitted answer sheet will be considered.

- After completing his National Service, Alex enrols at NTU to study Materials Engineering. In Prof. Leonardo’s thermodynamics class, Alex becomes fascinated by how energy and matter interact at the microscopic level. Inspired by this, he decides to develop a new computational tool—GibbsEngine—designed to manipulate Gibbs free energy for analysing advanced energy materials, such as high-performance batteries and solar cells. However, before Alex can fully implement his tool, he must first derive a key thermodynamic relationship using the Gibbs free energy equation.

- Help Alex build the foundation for his tool GibbsEngine by deriving the relationship:

dG=VdP-SdT

and prove one of the thermodynamic Maxwell relations that Prof. Leonardo would be proud of. Through this derivation, Alex will gain the necessary insight to fully control the Gibbs free energy framework in his research.									                                                               (10 Marks)

- Now that Alex has mastered the Maxwell relation, he applies GibbsEngine to a simple task in the lab: heating 600 mL of water to prepare his favourite instant noodles after a long day of research. To do this, he needs to heat the water from 20°C to 90°C at constant pressure (1 atm).

To help Alex, derive the change in entropy ΔS from the relation of heat δQ=mCp dT, given Cp​ for water 4.18 J/g·°C, and use it to calculate the change in entropy during the heating process from 20°C to 90°C.

[Hint: You do not need to use your previously derived Maxwell relation in (a).]

(10 marks)

- Given					 G=H-TS

dG=dH-TdS-SdT		 (1 marks)

From Enthalpy (H) equation,	H=U+PV,	 dH=dU+PdV+VdP 	(1 marks)

From the first law, dU=δQ-PdV and the second law δQ=TdS 		(2 marks)

Substituting, dH=TdS-PdV+PdV+VdP=TdS+VdP 		(1 marks)

Therefore, dG=dH-TdS-SdT=TdS+VdP-TdS-SdT

dG=VdP-SdT 				(0.5 mark)

Representing the G in terms of P and T:

G=G(P,T)

dG=(∂G∂P)TdP+(∂G∂T)pdT 			(1 marks)

Therefore,			 (∂G∂P)T=V; and (∂G∂T)p=-S 			(0.5 mark)

Find the cross derivatives,

(∂V∂T)P= [∂∂T (∂G∂P)T]P= ∂2G∂T∂P, and - ∂S∂PT= -[∂∂P (∂G∂T)P]T= -∂2G∂P∂T 	(2 marks)

For exact differentials, the cross derivatives are equal,

∂2G∂T∂P=∂2G∂P∂T				(0.5 mark)

Hence, -(∂V∂T)P= ∂S∂PT							(0.5 mark)

- Given:

Mass of water: 600 g (0.6 kg)

Initial temperature, Ti : 25°C = 298.15 K

Final temperature, Tf : 90°C = 363.15 K

Cp for water = 4.18 J/g·°C = 4180 J/kg·K

Derive and use the formula:

ΔS=TiTfm.cpTdT

Now, to calculate the change in entropy as Harry heats the water, we start with the specific heat capacity at constant pressure:

dQ=m⋅cp​⋅dT

The change in entropy is defined as:

dS=dQT

(1 mark)

Substituting into dQ=m⋅cp⋅dT we get:

dS=m⋅cp⋅dTT​

(2 marks)

To find the total change in entropy over the temperature range from Ti​ to Tf, we integrate:

ΔS=TiTfm⋅cp⋅dTT

​										(2 marks)

Since the mass m and specific heat capacity​ are constants, they can be factored out:

ΔS=m⋅cp⋅TiTf1TdT

(1 mark)

This integral gives:

ΔS=m⋅cp⋅lnTfTi

Now, substitute the known values:

- m=0.6 kg

- cp=4180 J/kg.K

- Ti=298.15 K

- Tf=363.15 90°C.

ΔS=0.6×4180×ln363.15298.15

ΔS=0.6×4180×0.197=537 J/K

(4 marks)

- Now Alex is modelling a small-scale calcination experiment in Prof. Leonardo’s thermodynamics class. A sample of powdered calcium carbonate will be heated in a tube furnace. Alex must predict whether decomposition to calcium oxide and carbon dioxide is thermodynamically favored at the planned temperature of 1000 K (assuming standard pressure). Use the provided formation enthalpies and Cp(T) functions to calculate the Gibbs free energy at the planned temperature and predict whether the decomposition process is spontaneous.

(10 Marks)

[Hint: You need to compute ​∆Gr, 1000=​∆Hr, 1000−1000​∆Sr, 1000 and predict whether the reaction is spontaneous based on the sign of ΔGr_1000∘​]

| Element/Compound | Standard Enthalpy (kJ/mol) |
| --- | --- |
| CaCO3 | -1206.9 |
| CaO | -635.1 |
| CO2 | -393.5 |

| Element/Compound | Standard Entropy (J/mol·K) |
| --- | --- |
| CaCO3 | 92.9 |
| CaO | 39.75 |
| CO2 | 213.74 |

| Material | a | b x 10-3 T |
| --- | --- | --- |
| CaCO3 | 92 | 1.5 |
| CaO | 43 | 1 |
| CO2 | 37.1 | 8.5 |

- Determine the enthalpy

​∆Hr, 1000 = ∆Hr,  298+2981000cp(CaO)dT+2981000cp(CO2)dT-2981000cp(CaCO3)dT

(2 mark)

= [(-635.1)+(-393.5)-(-1206.9) ] x 103 + ∆a2981000dT+ ∆b2981000TdT         (J/mol)

=178300 + (43+37.1-92)(1000-298)+ 1+8.5-1.5x 10-32(10002-2982)     (J/mol)

=178300 + (-11.9) x 702 + 0.004 x 911196

=173590.98 J/mol =173.6 kJ/mol

(2 marks)

- Determine the entropy

​∆Sr, 1000 = ∆Sr,  298+2981000[cpCaO+cpCO2-cp(CaCO3)] /T dT

(2 marks)

= (39.75+213.74-92.9) + 2981000[(43+37.1-92)/T+(1+8.5-1.5) x 10-3] dT

=160.59 + 2981000(-11.9T+0.008) dT

=160.59 -11.9 ln1000298+0.008(1000-298)

=151.80 J/molK

(2 marks)

- Gibbs Free Energy at 1000 K

​

∆Gr, 1000=​∆Hr, 1000−1000​∆Sr, 1000

= 173.6 kJ/mol – 1000K *151.80 J/molK

= 21.8 kJ/mol

(1 mark)

At 1000 K, ΔG>0. Decomposition is not spontaneous at standard conditions.

means calcination of CaCO₃ is thermodynamically unfavorable at 1000 K under standard CO₂ pressure (1 atm)

(1 mark)

- A gold block at 300 K is dropped from a height onto a perfectly rigid surface. Assume that all the block’s potential energy on falling is converted into kinetic energy (no rebound, no energy loss to the surface, and negligible heating of the surface). The block’s melting point is 1356 K, and its molar heat of fusion is 13 kJ/mol. The molar heat capacity of copper can be approximated as Cp=22.7+5.3×10−3T. Atomic weight of Au = 196.97.

Find the minimum height from which the block must fall to just begin melting upon impact. Explain why also, this is a highly unlikely scenario.

(12 Marks)

Consider the bullet as a system:

The appropriate First Law equation:

dQ + dW = dU + d(PE) + d(KE)

(2 marks)

No energy loss to the surface, and negligible heating of the surface, potential energy all transfers to kinetic energy:

d(KE) = mgh

(1 mark)

No heat flow in or out of the system, dQ=0. No work done on or by the copper, i.e. dW = 0. The copper melted so there is change in internal energy.

U + mgh = 0 or U = -mgh

(2 marks)

Also     U = H (J/kg) *m (kg) (at constant pressure)

ΔH_total = ΔH_sensible + ΔH_fusion

(1 mark)

To calculate the sensible heating:

ΔH_sensible = 28605.35 J/mol

(2 marks)

thus   ΔH_total = 28605.35 + 13000 = 41605.35 J/mol

(1 mark)

Total energy required for internal energy change:

U = ΔH_total *n = ΔH_total *m/M

mgh = ΔH_total *m/M

(2 marks)

thus h= ΔH_total/(M*g) =41605.35 J/mol/(196.97 g/mol*9.81*10^-3 N/g) = 21532 m

(0.5 mark)

This is highly unlikely because 21.5km above sea level is far too high for a gold block to be dropped from to melt (0.5 mark)

- An aircraft jet engine operates as a heat engine, converting fuel combustion heat into useful work to generate thrust.

(a) Write down the expression for the maximum possible efficiency of a heat engine (Carnot efficiency) in terms of the high-temperature reservoir TH and low-temperature reservoir TL

(1 marks)

- Explain how the efficiency of a jet engine can be increased. In your explanation, consider both thermodynamic factors (e.g., TH,TL) and engineering improvements (e.g., compression ratio, turbine design, material selection). Additionally, explain the possible constraints you might face.

(5 marks)

- Discuss the major practical limitation that prevents a jet engine from ever reaching 100% efficiency.

(2 marks)

### Start from definition of thermal efficiency:

η=WQH=1-QLQH

For a reversible Carnot cycle:

QLQH=TLTH

Hence:
ηCarnot=1-TLTH

Also  TH- TLTH is correct

(1 mark)

- Thermodynamic factors:

Increase TH ​: raise combustion temperature (within material limits). (1 mark)

Decrease TL ​: reduce exhaust temperature (limited by ambient air). (1 mark)

Engineering design factors:

Higher compression ratio → higher TH ​ relative to TL​. (1 mark)

Advanced turbine & compressor designs reduce irreversibilities /frictional losses. (1 mark)

Use of high-temperature-resistant materials & cooling methods allows higher combustion temperatures. (1 mark)

- Major practical limitation (2 marks)

To achieve 100% efficiency:

ηCarnot=1-TLTH​​ →1 requires TL​=0 K

Absolute zero is unattainable (3rd law of thermodynamics). (1 mark)

Additionally, increasing TH​ is constrained by material melting points, thermal stresses, and cooling limits. (1 mark)