SCHOOL OF MATERIALS SCIENCE & ENGINEERING

CONTINUOUS ASSESSMENT 1 SEMESTER 1 2023-2024

MS1016 – THERMODYNAMICS OF MATERIALS

(Question Sheet)

26 September 2023	Time Allowed: 1 hour

INSTRUCTIONS

1.	This paper contains THREE (3) short questions and TEN (10) MCQ questions.

2.	Answer ALL questions.

3. 	Write all answers on the answer sheet provided and clearly write your name and matriculation number on it.

4.	This is an open-notes test.

5.	All questions have ONLY ONE correct answer for the MCQ questions.

6.	You may retain this question sheet. Only answers on the submitted answer sheet will be considered.

PART I – SHORT QUESTIONS

- Show that dH = TdS + VdP for a system that does work reversibly.

(20 marks)

2.	Thor, the God of Thunder, has taken a brief sabbatical from Asgardian duties to study thermodynamics at NTU as your classmate in Prof. Leonardo’s class. Intrigued by the lessons, Thor wonders if the principles of thermodynamics could be applied to Mjölnir, his legendary hammer. The core of Mjölnir is a closed thermodynamic system that can be described by the Helmholtz free energy F(T,V). Subsequently, he has discovered through Prof. Leonardo’s lessons that his ability as a god to manipulate thunder actually changes the entropy within Mjölnir.

- Define the Helmholtz free energy as it pertains to Mjölnir. Derive its differential form and how it relates to the First law of thermodynamics. (5 marks)

- Starting from the Helmholtz free energy of Mjölnir, derive one of Maxwell's equations. Break down how this equation might allow Thor to understand the manipulation of entropy within Mjölnir when he uses his thunder powers. (10 marks)

- Relate the implications of a positive and a negative ΔF within Mjölnir on Thor's combat effectiveness. (5 marks)

3.	You are a resourceful scientist who has survived a shipwreck and along with a few others. Stranded on a deserted island with limited supplies, you realise that conventional cooking methods are not feasible. Amidst the wreckage, you find a chicken and ponder how to cook it. Having taken Prof. Leonardo’s Introduction to Thermodynamics class, you remember a theoretical discussion about how Kinetic Energy can transfer heat and that you could potentially cook a chicken through slaps and decide to explore this unconventional method as a last resort. You make a series of assumptions: an average human hand mass of 0.4 kg, a slap velocity of 5 m/s at the point of impact, and an energy transfer efficiency of only 10%. The chicken weighs 1.5 kg, has a specific heat of 2.8 J/g°C, and needs to go from an initial temperature of 10°C to a safe-to-eat temperature of 75°C.

How many slaps do you need to cook the chicken?

Make sure you discuss possible limitations to your method. 			(20 marks)

PART II Multiple Choice Questions

Answer all the TEN MCQ questions below. There is only ONE answer for each question. (4 marks each)

# "In a closed system where no energy is added or removed, the entropy of the system will not decrease." This statement is best described by:

# A) First Law of Thermodynamics

# B) Conservation of Mass

# C) Third Law of Thermodynamics

# D) Zeroth Law of Thermodynamics

# E) Second Law of Thermodynamics

- The efficiency of a heat engine can be best improved by:

A) Lowering the temperature at which energy is received

B) Balancing the temperature between input and output energies, so ΔT = 0

C) Raising the temperature at which energy is received

D) Lowering the operating temperature of the heat engine

E) Keeping the temperature of the heat engine constant

- A monoatomic ideal gas is contained in a cylinder that is in adiabatic conditions. The pressure of the gas drops from 30 atm to 10 atm, and the initial temperature is 400 K. What is the final temperature of the gas?

A) 205 K

B) 220 K

C) 226 K

D) 245 K

E) 260 K

- Which of the following process may be described as the Joule-Thompson effect?

A) Isentropic
B) Isenthalpic
C) Isothermal
D) Isochoric
E) Isobaric

- Consider the following processes for an ideal gas in a thermodynamic system:

(a) Process where both temperature and volume remain constant.

(b) Process that is reversible and adiabatic.

(c) Process at constant pressure.

(d) Process at constant volume.

(e) Process at constant temperature.

Match each process with its appropriate descriptor:

Descriptors:

(A) Isothermal

(B) Isochoric

(C) Isentropic

(D) Isobaric

(E) Adiabatic

Choose the lettered option that best matches the sequence (a,b,c,d,e):

A) B, C, D, A, E

B) B, C, D, E, A

C) C, A, B, D, E

D) D, A, B, C, E

E) B, C, D, B, A

- Consider a thermodynamic system characterized by the enthalpy H. Which of the following Maxwell's relations is valid for this system?

- ∂P∂S​T​=-∂T∂V​P

- ∂V∂ST​=∂T∂P​V

- ​∂V∂T​S​=∂S∂P​V

- ​∂P∂T​S​=-∂S∂V​P

- ∂S∂PT​=∂V∂T​P

- Consider an ideal gas defined by its enthalpy ( H ). For this system, the molar isothermal compressibility at constant temperature (T) is given as ∂S∂PT=0.4 J/(mol·K·Pa) and the molar coefficient of thermal expansion at constant pressure (P) is ∂V∂TP=1.2×10-6 m3/mol·K. Calculate the change in molar volume when the temperature of the system changes by 25 K during an isobaric (constant pressure) process. (Hint: Use the relations from the previous question)

- 2.5 ×10-7   m3/mol

- 3.0 ×10-7   m3/mol

- 6.0 ×10-7   m3/mol

- 1.2 ×10-6    m3/mol

- 2.4 ×10-6   m3/mol

- A cylinder contains a monoatomic ideal gas at an initial temperature of 300 K and an initial pressure of 2 atm. The gas undergoes adiabatic compression, and the final pressure is 8 atm. Considering Cp​=52R, what is the final temperature of the gas?

- 1200 K

- 689 K

- 700 K

- 1010 K

- 900 K

- Which of the following is NOT a property of an ideal gas?

    - The internal energy is not a function of the volume

    - The enthalpy is not a function of the pressure

    - PV = nRT

    - CV–CP=R

    - No intermolecular forces

- Which of the following statements best describes the concept of the "heat death" of the universe according to thermodynamic principles?

  - The universe will eventually reach a state of minimum entropy.

  - The universe will eventually contract into a singularity, releasing an enormous amount of heat.

  - The universe will eventually reach a state of maximum entropy where all matter is evenly distributed.

  - The universe will eventually reach thermal equilibrium but with decreasing entropy.

  - The universe will eventually collapse, leading to a decrease in overall entropy and increase in free energy.